INF5350

Obligatory exercises following lecture #5

Please submit latest 11-Oct-2019 to Tohid (tohidm@ifi.uio.no)

- Explain the difference between temporal noise and fixed pattern noise

- Give an example of FPN that is signal dependent

- If dark FPN (at zero light) represents fixed offsets in a sensor, then how is it possible that the FPN average offset value across the whole image is zero?

- List the temporal noise sources in CMOS image sensors

- Can you think of a temporal noise source that appears in the image as horizontal lines?

- A sensor is illuminated with 400kphotons/s/pixel. Camera exposure time is 1ms. What is the signal/noise ratio at the pixel input? What is the rms noise?

- When the light level onto a sensor increases, then the input noise increases, i.e. the histogram of #photons gets wider. Nevertheless, the SNR is improving. Explain why.

- Propose a method to compensate (remove) PRNU in a camera

- Calculate S/N ratio at positions A, B, C, and D in below figure. Assumptions:

  - PD has accumulated 10 photo-electrons and 5 dark current electrons

  - Transfer gate (TX) lag is zero

  - Conversion gain, CG=90uV/e-

  - Source follower gain, G=0.85

  - Source follower noise, Nsf=200uV rms

  - ADC is 12bit, Vref is 1V, and ADC noise is dominated by quantization noise